# IDS Detection of Nmap Decoy Scanning: A Home Lab Case Study

## Objective

Build an isolated three-machine lab to observe how a network-based Intrusion Detection System (Suricata) responds to reconnaissance traffic, specifically comparing a plain SYN scan against an Nmap decoy scan (-D). The goal was to answer a practical question: when an attacker hides among spoofed decoy source IPs, what does the defender actually see — and how can they still tell what's real?

## Lab Architecture

| Role | OS | Hostname | Static IP | Purpose |
| --- | --- | --- | --- | --- |
| Attacker | Kali Linux | attacker | 192.168.56.10 | Runs scans, generates spoofed traffic |
| Target | Kali Linux | tariq | 192.168.56.20 | Scan target, runs a listener service |
| Sensor | Debian | IDS | 192.168.56.50 | Runs Suricata, captures and analyzes traffic |

Network: VirtualBox Internal Network (intnet), fully isolated from the host and any external network — no traffic ever left the virtual segment, which matters given the lab involves spoofed packets.

Sensor dual-adapter setup: The sensor VM required a second adapter (NAT) purely for package installation and rule updates, since Internal Network by design has no DHCP or DNS and therefore no internet access. Adapter 1 (Internal Network, static 192.168.56.50) handled all lab traffic capture; Adapter 2 (NAT) handled apt and suricata-update only.

## Tooling

| VM | Tools Installed |
| --- | --- |
| Attacker | nmap, hping3, macchanger, scapy (python3-scapy) |
| Target | openssh-server, ad hoc nc -lvp 8080 listener |
| Sensor | suricata 7.0.10, Emerging Threats Open ruleset (~52,000 rules) |

## Suricata Configuration

HOME_NET was scoped to the lab subnet:

HOME_NET: "[192.168.56.0/24]"

Key finding: the default Emerging Threats Open ruleset does not include a generic behavioral "port scan" rule. It's signature-based — built to match known attack patterns, not raw scan volume. A plain nmap -sS against an uncommon high port generated zero alerts against the stock ruleset, even though Suricata was capturing every packet correctly.

To detect scanning behavior directly, a custom threshold rule was added to local.rules:

alert tcp any any -> $HOME_NET any (msg:"LOCAL SCAN Possible Port Scan Detected"; flags:S; threshold: type both, track by_src, count 5, seconds 10; sid:1000001; rev:1;)

This fires when any single source IP sends 5 or more SYN packets to the lab subnet within a 10-second window — a reasonable proxy for scan behavior regardless of which ports are targeted.

local.rules had to be explicitly added to the rule-files list in suricata.yaml and placed in Suricata's actual default rule path (/var/lib/suricata/rules/, not the more intuitive /etc/suricata/rules/) before it was picked up.

## Test 1: Baseline SYN Scan (No Evasion)

sudo nmap -sS 192.168.56.20

Result: Suricata alerted immediately and correctly.

{

"event_type": "alert",

"src_ip": "192.168.56.10",

"dest_ip": "192.168.56.20",

"alert": {

"signature": "LOCAL SCAN Possible Port Scan Detected",

"severity": 3

}

}

One alert, one source IP, correctly attributed to the real attacker. This confirmed the full pipeline — capture, rule matching, and alerting — was working end to end before introducing evasion.

## Test 2: Decoy Scan

sudo nmap -D 192.168.56.100,192.168.56.101,ME,192.168.56.102 -sS 192.168.56.20

This instructs Nmap to interleave the real scan with spoofed-source packets appearing to originate from three additional (nonexistent) hosts, so the target sees four apparent sources scanning it simultaneously.

Result: Suricata generated alerts attributing scan activity to the decoy IPs as well as the real attacker — for example:

{

"event_type": "alert",

"src_ip": "192.168.56.102",

"dest_ip": "192.168.56.20",

"alert": {

"signature": "LOCAL SCAN Possible Port Scan Detected",

"severity": 3

}

}

From the alert log alone, all four source IPs (.10, .100, .101, .102) appear equally credible as the origin of the scan. The IDS has no built-in way to distinguish the real attacker from the decoys — it can only report what it observes at the packet level, and spoofed packets are, at that level, indistinguishable from real ones.

## Cutting Through the Ambiguity: ARP Verification

Alert data alone wasn't enough to identify the real attacker, so the next step was checking the target's ARP table immediately after the scan:

# on the target VM

arp -a

? (192.168.56.101) at <incomplete> on eth0

? (192.168.56.102) at <incomplete> on eth0

? (192.168.56.100) at <incomplete> on eth0

? (192.168.56.10)  at 08:00:27:a4:c4:3b [ether] on eth0

Only 192.168.56.10 resolved to an actual MAC address. The three decoy IPs show <incomplete> — the target attempted to ARP for them (because it saw packets claiming those source addresses) but received no reply, since no real device exists at those addresses.

This is the key defensive insight: decoy scanning can convincingly spoof Layer 3/4 packet headers, but it cannot fake a live device answering an ARP request at Layer 2. A completed ARP resolution — or, more generally, a completed TCP handshake — is a much stronger signal of a genuine live host than anything in an IDS alert log alone.

## Key Takeaways

- Signature/threshold-based IDS rules will treat spoofed and real sources identically. Nothing in a Suricata alert distinguishes a decoy from a real attacker — both look like a source IP that sent a scan-pattern of packets.

- Default rulesets often lack generic scan detection. A custom threshold rule was necessary to detect scanning behavior at all, since the stock ET Open ruleset is signature-driven rather than behavior-driven.

- ARP (or completed handshakes generally) is a practical way to separate real from spoofed sources during an active investigation, since spoofed IPs can't complete a two-way exchange without a real device behind them.

- Network isolation matters. Running this on a VirtualBox Internal Network kept all spoofed/scanning traffic fully contained, with no risk of it touching a real network or device outside the lab.

## Setup Notes / Lessons Learned

A significant part of this exercise was troubleshooting the lab environment itself — worth recording since these are common, easy-to-hit issues:

- auto and iface interface names must match exactly in /etc/network/interfaces. A single-character typo (emp0s3 vs enp0s3) silently prevented a static IP from being applied on boot, with no obvious error until traffic simply didn't route.

- VirtualBox Internal Network provides no DHCP or DNS. Any VM needing to install packages requires either a temporary switch to NAT/DHCP, or (better) a second adapter dedicated to NAT for internet access, while the primary adapter stays on Internal Network for isolated lab traffic.

- Promiscuous Mode must be set to "Allow All" on the sensor's adapter for Suricata to see traffic not addressed to its own MAC — without it, the sensor only sees broadcast and directly-addressed traffic.

- Suricata's default rule path is /var/lib/suricata/rules/, not the more intuitive /etc/suricata/rules/. Custom rules placed in the latter won't load unless copied to the correct location and referenced in suricata.yaml.

## Possible Next Steps

- Test additional evasion techniques (packet fragmentation, timing templates -T0–-T2, idle/zombie scans) against the same rule to see what does and doesn't trigger it

- Compare Suricata's behavior against Snort or Zeek on the same traffic

- Extend the custom rule set to catch fragmentation-based evasion specifically

- Test MAC address spoofing (via macchanger, already installed on the attacker) and observe what, if anything, is visible at the network layer versus the IDS layer